‘The Astronomy Files’ Hack Club Half Life Lock in

| Designator | Component Class | Core Description | Example Manufacturer Part Number (MPN) | Footprint / Package | Qty | Sourcing Notes |
| --- | --- | --- | --- | --- | --- | --- |
| U1 | Microcontroller | ESP32-S3 Wi-Fi & BLE Module, 4MB Flash | ESP32-S3-MINI-1-N4 | SMD Module (65-pin) | 1 | Primary MCU. Handles deep-sleep and Wi-Fi NTP/API quote fetching. |
| U2 | Real-Time Clock | High-Accuracy I2C RTC with integrated TCXO | DS3231MZ+TRL | SOIC-8 (150 mil) | 1 | Keeps time tracking accurate to ±2 minutes per year, even if the main battery dies. |
| U3 | Battery Charger | Linear Li-Ion Battery Charger with thermal protection | TP4056 | SOP-8-PP | 1 | Standard open-source friendly chip to safely manage 3.7V LiPo charging via USB. |
| U4 | Voltage Regulator | 3.3V 600mA Low-Dropout (LDO) Regulator | AP2112K-3.3TRG1 | SOT-23-5 | 1 | Steps down battery voltage to a stable 3.3V for the ESP32 and E-Paper panel. |
| DISP1 | Display Panel | 1.54-inch 200x200 Electronic Paper Display | GDEY0154D67 | FPC-24 Pin Connection | 1 | Zero-power static state. Connects via SPI interface. |
| BATT1 | Battery Source | 3.7V 500mAh Lithium Polymer Battery | Generic 503035 LiPo Cell | JST-PH 2.0mm / Leads | 1 | Slim profile battery packed under the PCB board inside the pocket case. |

| Component Designator | Core Part | Estimated Unit Price | Sourcing Platform Variant |
| --- | --- | --- | --- |
| U1 | ESP32-S3-MINI-1-N4R2 | £3.38p | LCSC (DigiKey runs ~£4.05) |
| U2 | DS3231MZ+TRL RTC | £2.29p | LCSC (Mouser/DigiKey brands run ~£7.40+) |
| U3 | TP4056 Battery Charger | £0.07p | LCSC (Pre-soldered breakout modules run ~£0.99) |
| U4 | AP2112K-3.3TRG1 LDO | £0.20p | Mouser / DigiKey |
| DISP1 | 1.54" E-Paper Screen | £2.44p | Elecrow / AliExpress (Waveshare panels run ~£9.99) |
| BATT1 | 3.7V 500mAh LiPo Cell | £5.99p | eBay / AliExpress |
| --------------------- | ---------------------------- | ------------------ | ------------------------------------- |
| EST. TOTAL | All Core Hardware Components | £14.92 YAY DOABLE | Excludes raw PCB manufacturing & basic passives |

‘The Astronomy Files’ Master System Pin Map Thing

1. Real-Time Clock (I2C Bus) — Time & Date

- DS3231 Pin 5 (SDA) ──► ESP32-S3 GPIO 8 (Pull high with a 4.7kΩ resistor to 3.3V)

- DS3231 Pin 6 (SCL) ──► ESP32-S3 GPIO 9 (Pull high with a 4.7kΩ resistor to 3.3V)

2. E-Paper Screen (SPI Bus) — Visual Output (Quotes & Counters)

- DISP1 BUSY ──► ESP32-S3 GPIO 4 (Input pin to check if display is updating)

- DISP1 RST (Reset) ──► ESP32-S3 GPIO 5

- DISP1 D/C (Data/Command) ──► ESP32-S3 GPIO 6

- DISP1 CS (Chip Select) ──► ESP32-S3 GPIO 7

- DISP1 SCLK (Clock) ──► ESP32-S3 GPIO 12

- DISP1 DIN (MOSI Data) ──► ESP32-S3 GPIO 11

3. Wi-Fi Connection — Daily Astro-Quotes

- ESP32-S3 Internal Antenna ──► 2.4GHz Local Network (Wakes every 24 hours to fetch and cache a fresh quote via JSON API, then enters deep sleep)

4. Processing Loop — Cosmic Math Calculations

- Internal Firmware Engine ──► 64-bit Floating Registers (uint64_t

) (Pulls Unix time from the DS3231 over I2C and adds it to the baseline constants: ~13.787 Billion Years since the Big Bang and ~4.5684 Billion Years since Solar System formation)